TRAIL ROUTE:  S-1 (Matadero Creek/Page Mill Trail)

| TRAIL OPTIONS | ## SEGMENTS OF THE PROPOSED TRAIL OPTIONS | APPROX LENGTH (miles) | ## EXISTING STATUS |  |  | ###### TRAIL CONDITION |  |  | ## CONSTRAINTS (Environmental/Public/Agency Concerns) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Completed | Partially-completed | None | Dirt Path | Paved | Other | Biological Resources (i.e. Special Species) | Hydrology and Water Quality (i.e. adj. creek) | Existing Use Restrictions (i.e. No bikes) | Concerns of the Public/ Landowners | Other |
| ### S1-A: Matadero Creek Alignment Option (as per Santa Clara County Countywide Trails Master Plan Update) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #### Segment 1 (S1-A1) | From Page Mill Rd to Foothill Expwy to Old Page Mill Rd | .2 miles | X | - | - |  | 7 ft asphalt (north) | Within CountyROW | In special conservation area | - | - | - | Traffic concerns- Access from pedestrian/ bike facilities could be difficult |
| Segment 2 (S1-A2) | From Old Page Mill Rd to Page Mill Rd | .9 miles | X | - | - | - | 24 ft County road |  | In special conservation area | Adjacent creek | - | Access road for LAH residences/ SU agric. tenants | Low traffic volumes Possible easement on County roada |
| Segment 3 (S1-A3) | From Page Mill Rd to Arastradero Rd | .6 miles | - | - | X | - | - | - | - | - | - | - | Traffic concerns-2 fwy on-ramps; Room in R.O.W. for bike/ pedestrian facilities |
| Segment 4 (S1-A4) | From Arastradero Rd to Arastradero Staging Area | .5 miles | - | - | X | - | - | - | - | - | - | - | Bike lanes on Page Mill Rd in LAH; none on Arastradero Rd |
| ### S1-B: Stanford Avenue/City of Palo Alto Trails Connection/Alignment Options |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #### Segment 1 (S1-B1) | From Stanford Ave/Bowdoin St to Page Mill Rd on Peter Coutts Ln | .6 miles | X |  |  |  | 6 ft asphalt | Stanford Green belt | - | - | No horses permitted | Adjacent to Stanford University residences | Existing path being used primarily by residents and visitors |
| Segment 2 (S1-B2) | From Peter Coutts Ln/Page Mill Rd to JSB on Page Mill | .5 miles | X |  |  |  | 6 ft asphalt | Stanford Green belt |  |  | No horses permitted | Adjacent to Stanford University residences | Existing path being used primarily by residents and visitors |
| Segment 3 (S1-A1) | Connects to S1-A1 | - |  |  |  |  |  |  |  |  |  |  |  |
| OR Option #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Segment 1A (S1-B1A) | From Stanford Ave/Bowdoin St to Raimundo Wy | .6 miles | X |  |  |  | 6-8 ft asphalt | Stanford Green belt |  |  | No horses permitted | Adjacent to Stanford University residences | Need to cross Stanford Avenue |
| #### Segment 2A (S1-B2A) | From Raimundo Wy to JSB | .3 miles | X |  |  |  | 6-8 ft asphalt | Stanford Green belt |  |  | No horses permitted | Adjacent to Stanford University residences | Need to cross Stanford Avenue |
| Segment 3 (S1-B3A) | Along JSB to Page Mill/Old Page Mill | .5 miles |  |  | X |  |  |  |  |  |  | San Francisco Water District Pipeline R.O.W. |  |
| Segment 4 (S1-A1) | Connects to S1-A1 | - |  |  |  |  |  |  |  |  |  |  |  |

TRAIL ROUTE:  S-1 (Matadero Creek/Page Mill Trail) – Continued

| TRAIL OPTIONS | ## TRAIL SEGMENT (from ______ to _____) | ### COUNTYWIDE TRAILS MASTER PLAN: ### (Goals for a Sub-regional Trail Route) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Will the trail segment, if built, provide regional recreation benefits? | Will the trail segment, if built, provide regional transportation benefits? | Will the trail segment provide continuous and convenient linkages between City and County trails? | Will the entire trail route link two or more Regional Trails? | Are Other Agencies Involved or Acquisitions Needed? | CURRENT CONDITIONS vs. INTENDED USES: Does the current alignment allow for the intended uses of the Countywide Trails Master Plan? |  |  |  |
|  |  |  |  |  |  |  | Hiking | Biking (on-road) | Biking (off-road) | Equestrian-riding |
| S1-A: Matadero Creek | S1-A1: From Page Mill Rd/Foothill Expwy to Old Page Mill Rd | Yes | Yes | No |  | SCC Roads & Airport | X | X | X | Not intended |
|  | S1-A2: From Old Page Mill Rd to Page Mill Rd | Yes | Yes | No |  | SCC Roads & Airport | X (w/in ROW) | X | No | Not intended |
|  | S1-A3: From Page Mill Rd to Arastradero Rd | No | Yes | No |  | CALTRANS, Los Altos Hills | No | X | No | Not intended |
|  | S1-A4: From Arastradero Rd to Arastradero Staging Area | Not applicable | N/A | N/A |  | Los Altos Hills, Palo Alto | No | X | No | Possible (not on SU land) |
| S1-B1: Stanford Ave (Option 1) | S1-B1: From Stanford Ave/Bowdoin St to Page Mill Rd on Peter Coutts Ln | Yes | Yes | Yes |  | Palo Alto | X | X | X | Not intended |
|  | S1-B2: From Peter Coutts Ln/Page Mill Rd to JSB on Page Mill Rd | Yes | Limited | Yes |  | Palo Alto | X | X | X | Not intended |
|  | S1-B3: Connects to S1-A1 |  |  |  |  |  |  |  |  | Not intended |
| S1-B1A: Stanford Ave (Option 2) | S1-B1A: From Stanford Ave/Bowdoin St to Raimundo Wy | Yes | Yes | Yes |  | Palo Alto | X | X | X | Not intended |
|  | S1-B2A: From Raimundo Wy to JSB | Yes | Limited | Yes |  | Palo Alto SCC Roads & Airport | X | X | X | Not intended |
|  | S1-B3A: Along JSB to Page Mill/Old Page Mill Rd | Yes | Limited | Yes |  | Palo Alto SCC Roads & Airport | X | X | X | Not intended |
|  | #### Connects to S1-A1 |  |  |  |  |  |  |  |  | Not intended |

TRAIL ROUTE:  C-1 (San Francisquito/Los Trancos Trail)

| TRAIL OPTIONS | ## SEGMENTS OF THE PROPOSED TRAIL OPTIONS (from ___ to ___) | APPROX LENGTH (miles) | ## EXISTING STATUS |  |  | ###### TRAIL CONDITION |  |  | ## CONSTRAINTS (Environmental/Public/Agency Concerns) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Completed | Partially-completed | None | Dirt Path | Paved | Other | Biological Resources (i.e. Special Species) | Hydrology and Water Quality (i.e. adj. creek) | Existing Use Restrictions (i.e. No bikes) | Concerns of the Public/ Landowners | Other |
| ### C1-A: San Francisquito/Los Trancos Alignment Option |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #### Segment 1 (C1-A1) | From El Camino Real to Arboretum Rd | .4 | X | - | - | - | 6 ft asphalt adj. Sand Hill Rd | - | - | - | - | - | Existing bike lane on Sand Hill Road |
| Segment 2 (C1-A2) | From Arboretum Rd to Pasteur Dr | .5 to .6 | X | - | - | 10 ft top of creek bank/ | 6 ft asphalt to Sand Hill Rd | Not improved | - | Yes (adjacent creek corridor) | - | - | Dirt path in the Archeological Preserve adjacent to creek, part of SCVWD maintenance road |
| Segment 3 (C1-A3) | From Pasteur Dr to Searsville Path | .5 | X | - | - | - | 6 ft asphalt to Sand Hill Rd | - | - | - | - | - | - |
| Segment 4* (C1-A4) | From Searsville Path to Sand Hill Rd/Santa Cruz Ave | .4 | - | X | - | - | - | - | - | - | - | - | 6 ft paved path planned but not approved by City of Menlo Park To be evaluated: Alternate route within Stanford campus will be 3-4 X the length of this segment |
| Segment 5* (C1-A5) | From Sand Hill Rd /Santa Cruz Ave to Alpine Rd | .2 | X | - | - | 6 ft asphalt | - | - | - | Yes (adjacent to San Francisquito Creek corridor) | - | - | Not accessible from all parts of the existing intersection |

TRAIL ROUTE:  C-1 (San Francisquito/Los Trancos Trail) - Continued

| TRAIL OPTIONS | ## TRAIL SEGMENT (from ______ to _____) | ### COUNTYWIDE TRAILS MASTER PLAN: ### (Goals for a Sub-regional Trail Route) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Will the trail segment, if built, provide regional recreation benefits? | Will the trail segment, if built, provide regional transportation benefits? | Will the trail segment provide continuous and convenient linkages between City and County trails? | Will the entire trail route link two or more Regional Trails? | Are Other Agencies Involved or Acquisitions Needed? | CURRENT CONDITIONS vs. INTENDED USES: Does the current alignment allow for the intended uses of the Countywide Trails Master Plan? |  |  |  |
|  |  |  |  |  |  |  | Hiking | Biking (on-road) | Biking (off-road) | Equestrian-riding |
| C1-A: San Francisquito/ Los Trancos | C1-A1: From El Camino Real to Arboretum Rd | Yes | Yes | Yes |  | Palo Alto | X | X | X | Not intended |
|  | C1-A2: From Arboretum Rd to Pasteur Dr | Yes | Yes | Yes |  | Palo Alto | X | X | X | Not intended |
|  | C1-A3: From Pasteur Dr to Searsville Rd | Yes | Yes | Yes |  | Palo Alto | X | X | X | Not intended |
|  | C1-A4: From Searsville Rd to Sand Hill Rd/Santa Cruz Ave | Yes | Yes | Yes |  | - | - | - | - | Not intended |
|  | C1-A5: From Sand Hill Rd /Santa Cruz Ave to Alpine Rd | Yes | Yes | Yes |  | Paths mnted by Menlo Park | X | No | X | Not intended |